Title: Impact of Muscle Coactivation on Metabolic Cost and Task Performance in Stabilizing Arm Posture During Mechanical Perturbations.

Abstract

Muscle coactivation refers to the simultaneous activity of agonist and antagonist muscles. Historically, this phenomenon has been understood as stabilizing the body by making muscles stiff and resistant to motion. Recent observations suggest that coactivation may make the nervous system more responsive to sensory feedback. Muscle activity is costly, however, and we know little about the metabolic cost associated with coactivation. This study investigates the trade-off between metabolic cost and task performance under different levels of muscle coactivation. Five participants attempted to maintain a fixed arm posture in a Kinarm exoskeleton robot while encountering mechanical disturbances. The disturbances rapidly extended or flexed the elbow and displaced the participant’s arm from the target. They first performed the task under self-selected levels of muscle coactivation. Biofeedback was then used to maintain a level of coactivation corresponding to [0.5, 2, 4, 8, 16]x their self-selected (1x) level. The metabolic cost was measured with gas respirometry, the activity of upper limb muscles using surface EMG, and behavioral responses using success rates and peak elbow displacements. We found that muscles became more responsive to the disturbances, improving success rates but increasing metabolic costs at higher coactivation levels. At the self-selected coactivation level, participants had a success rate of 30-50%, which increased to nearly 100% with higher coactivation. These findings suggest that higher coactivation levels enhance performance but increase metabolic cost, indicating that participants may prioritize metabolic cost at self-selected levels of muscle coactivation. This research offers insights into muscle coactivation's role in movement.